A. Exits *

1. Remove unapproved locks or latches from exit door(s)                  	2. Unlock all exit doors during business hours

3. Remove all obstructions from exits, corridors & stairways                4. Exit door and hardware to operate properly.

5. Means of egress shall be free of ice & snow.

6. Fire doors shall not be secured by unapproved  devices.

B. Exit Signage and Lighting *

7. Required exit signage shall be illuminated, primary and

secondary bulbs functioning.

8. Emergency lighting shall be in good working order.

9. Provide emergency lighting for exits, aisles,

corridors and stairways.

C.  Fire Protection Systems

10. Sprinkler Room shall be labeled.

11. Fire Alarm system to be inspected annually:

State ID# _____________________________.

12. Alarm system shall be operational & properly secured.*

13. Alarm stations to be unobstructed. *

14. Smoke detectors properly located.

15. Suppression Systems shall be inspected annually:

State ID # _____________________________.

16. Suppression system shall be operational. *

17. Suppression system shall be properly secured. *

18. Spare sprinkler heads and wrench on site:

Minimum of 6.  (For NFPA 13 systems)

19. 18" unobstructed clearance for sprinkler heads. *

20. All sprinkler heads free of dirt, grease, or paint.

21. FDC shall have protective caps & swivels in  place.

22. FDC shall be labeled and accessible. *

23. Fire Pump shall be inspected annually.

State ID# _____________________________.

D. Fire Extinguishers

24. Fire extinguishers shall be unobstructed. *

25. Fire extinguishers shall be inspected annually.

26. Fire extinguishers shall be fully charged.

27. Fire extinguishers shall be installed on hangers.

E. Kitchen Areas

28. Commercial hood systems shall be inspected semi-

annually.   State ID #_____________________.

29. Hood systems to be properly maintained, cleaned,

filters in place, protective covers over nozzles.

30. Appliances properly positioned under hood and

suppression nozzles. *

31. Manual pull station located near exit.

32. Appropriate fire extinguisher located by exit in kitchen.

33. GFCI for kitchen/bathroom outlets tested properly

F. Heating Plant         Boiler inspection:   yes   no    N/A

Oil Fired

34. Maintain clearance of 3' from combustibles. *

35. Remote shut off switch present near access door.

36. Manual shut off switch present.

37. Firematic device present.

F. Heating Plant

Gas Fired

38. Maintain clearance of 5' from combustibles. *

39. Manual shut off valve present.

All Heating Systems

40. Area enclosed by 1 hr. rating with self-closing door

and approved make-up air vent or limited area

sprinkler present.

41. Vent pipe secured and in good physical condition. *

42. Vent pipe has positive pitch.

G. Utilities

43. Elect. panel - maintain clearance of 5' from                              combustibles. *

44. Elect. panels shall have only approved hold open                     devices on circuit breakers.

45. Elect. panels and junction boxes shall have covers                   in place.

46. Outlets and switch plates shall have covers in place

and be in good condition.

47. Extension cords shall not be permitted.

48. Surge protectors shall be UL listed and not listed on                the CPSC recall sheet.

49. Use of multi-plugs is prohibited.

H. Housekeeping

50. Openings in fire barriers, ceilings and walls

shall be sealed with approved covering.

51. Use of approved containers for combustible waste.

52. Arrange storage areas in orderly manner to provide                 exiting for occupant(s) and firefighter access.

53. Reduce storage height to at least 2' below ceiling. *

54. No accumulation of combustible wastes on interior

or exterior of building.

55. Boiler, mechanical and electrical panel rooms shall

not be used for storage.

56. Transoms shall be fixed.

57. Move dumpsters a minimum of  5' from the building              and/or overhangs.

58. Secure dumpster or dumpster covers.

I. Flammable Liquids

59. Storage of flammable liquids in excess of 10    	                    gallons shall be in an approved cabinet.

60. Discontinue use of Class 1 (gasoline, etc.) liquids

for cleaning. *

61. Store liquids away from exits, aisles, corridors, and

stairways. *

J. Access

62. Address numbers shall be visible from the street

63. Numbers shall be a minimum of 4" in height with a

contrasting background

64. Access to driveway, access lanes shall be kept clear                 to a minimum of 13'6".

65. Knox box required for all fire alarm and       	                    suppression systems.

Asterisked (*) sub-sections &/or items require immediate correction action.